User Manual

AeroEase User Manual

Fig.1 The Overall Guide of Using AeroEase.（待添加）

The AeroEase Portable Nebulizer is a portable, all-in-one device for storing, nebulizing and delivering inhaled medication. Based on vibrating-mesh nebulization, it turns a liquid drug into a fine aerosol that is inhaled through the mouthpiece, and records every treatment on your phone. To facilitate our users' better application of AeroEase for inhaled asthma therapy, we have developed the following user manual.

Preparation

- Charge the rechargeable 18650 battery module before the first use.

- Check that the nebulizer plate and its rubber gasket sit flat in the slot, and that the hot-melt adhesive does not cover the nebulization area.

- Place the device upright on a level, hard surface.

- Take off the protective cap from the mouthpiece.

Nebulization

- Open the lid on the top of the device.

- Add the liquid medication prescribed by your physician into the upper reservoir.

- Close the lid.

- Insert the mouthpiece into the front port until the silicone sealing ring is fully seated.

- Press the button once to start nebulization.

- Sit upright, put the mouthpiece into your mouth and seal your lips around it.

- Breathe normally through your mouth; the holding chamber keeps the aerosol between breaths.

- Wait until the device stops by itself. Press the button again if you need to stop early.

- Take the mouthpiece out and put the protective cap back on when the treatment is finished.

App Usage

- Establish a connection: launch the mobile application, select your AeroEase device and keep Bluetooth on your phone enabled. The device connects over BLE.

- Data monitoring: the treatment time is recorded on the device automatically and uploaded to the app, where you can review your medication history together with your physician.

Cleaning and Storage

- Detach the holding chamber and the mouthpiece from the base.

- Rinse the parts that contact the medication with clean warm water.

- Drain the residual medication collected by the return structure at the base.

- Let all parts air-dry completely before you reassemble them.

- Wipe the base with a damp cloth only — it houses the electronic control system and must never be immersed in water.

- Put the protective cap back on and store the device upright when it is not in use.

Safety

- The medication compartment is separated from the electronic compartment, so a liquid leak is kept away from the circuit; check that the electrical compartment stays dry after every use.

- A rubber gasket and hot-melt sealing keep the liquid medication out of the electronics; stop using the device, remove the battery and let it dry if the compartment becomes damp.

- The ESP32 manages the operating state of the device, which prevents unintentional continuous operation.

- A silicone sealing ring forms an interference fit between the mouthpiece and the holding chamber, which prevents aerosol leakage during inhalation.（此条待定）

- Use only the liquid medication prescribed by your physician, and keep the device and the battery out of the reach of children.

- AeroEase is still at the prototype development stage and is not a certified medical device; its long-term stability, material compatibility and safety for medical applications require further verification.